Automatic Cell Outlining SOP:

- Create max projections of images using automatic max projection program, changing the folder name at the bottom of the program and the indicator list to include the nucleus and cytoplasm channels you want to use for outlining

- Launch app designer in MATLAB 2025 by typing “appdesigner” into the MATLAB terminal

- Open the Automatic_Outlining_GUI MATLAB app

- Press Run on top task bar

- Edit the “Nuc Channels” and the “Cyto Channels” (any fluorescence channel present in the cytoplasm)

  - All names of images must be the same other than these channel names and there must not be any duplicate names

- Click on “Load Folder” button, select folder that contains the max projections created in step 1

  - The first image will appear with the automatic segmentation already complete

- Adjust the “Num Levels”, “Border”, and “Edge Minimum” parameters in the GUI to get better segmentation

  - Increase the number of level curves if dimmer areas of the cell are not included in the cell outlines. Decrease the number of level curves if too much background is being included in the cell outlines.

  - Increase the value of the “Border” parameter to increase the border area around the image where nuclei are excluded (number of pixel border exclusion)

  - Decrease the value of “Edge Minimum” parameter to increase the number of vertices in cell outlines. Increase the value of “Edge Minimum” to decrease the number of vertices in cell outlines.

Note: “Num Cells” is only an indicator of the number of cells detected, adjusting this parameter has no effect on segmentation.

- Once happy with the overall segmentation you can fine tune your outlines and add additional nucleus and cell outlines.

  - Click on any cell or nucleus you do not want to include, they will turn red, then press “Delete Selected” (they will then be removed). You can delete multiple at once. Alternatively click on all cells and nuclei that you want to keep as outlines turning them red, then press “Delete Unselected” to delete all selected cell and nucleus outlines. If you click on a cell or nucleus thereby selecting it (turned red) click again to unselect. Once you have pressed delete you can not get the outlines back, but if you press segmentation again the original outlines will come back, and you can restart this process for this image. Note: the program requires you to have an equal number of cell and nucleus outlines (will produce an error warning and ask you to retry during saving).

  - Click on “Draw Cell” or “Draw Nuc” buttons to begin drawing a new nucleus or cell outline. Left click adds new vertices and right click adds final vertex. After each outline drawn you must click the button again to draw another.

  - Drag the vertices of any cell or nucleus outline you want to keep but might not be perfect.

- When happy with all the cell outlines for that image press save. A conformation message will appear.

- Press next to go to next image and repeat the process. If you want to go back to previous image press previous button. (Please note: if you want to re-save outlines for an image you have already saved outlines for go into the results folder and delete them first). Additionally, to advance directly to an image in the folder you can change the “Image Num” parameter to the image number you want to advance to.

- Open “SkeletonAndPrint” folder which will contain the binary masks for all cells and nucleus you outlined.

Please see the presentation in this folder for examples of different adjustments you can make with the automatic cell outlining GUI.